# Field Normalization

I loaded the data we received from Andrea. The data is of the format:

| X_1 | Y_1 | Z_1 | Ex_1 | Ey_1 | Ez_1 |
| --- | --- | --- | --- | --- | --- |
| ... | ... | ... | ... | ... | ... |
| X_N | Y_N | Z_N | Ex_N | Ey_N | Ez_N |

Each data file we have represents a mode. We distinguish between the data by some parameters: the frequency of the field, the radius of the device and Ef of the graphene.

The first step is to normalize the modes. It is done by calculating a numeric integral over the norm of the field E(r)2 at each point, and then diving the samples by the result in order to obtain the normalized functions uk(r):

EkrEk*rdr=A2

Ekr=Auk(r)

ukruk*rdr=1

The samples are given in Cartesian coordinates, and the integral is calculated over the volume of the cylindric device. It sounds like it would be easier to work with cylindric coordinates.

Calculating the integral might sound not so difficult, but let’s have a look at the histograms of the coordinates of the samples for the mode Ef=0.2eV, R=2.6μm, f=3.41THz:

And in Cartesian coordinates the situation isn’t better:

In this case, we can’t calculate the integral by summing as iρif(ρi,θi,zi)ΔρΔθΔz because the intervals at each axis are not uniformly distributed.

The method I used was:

- Split the space into tetrahedrons with our samples coordinates as vertices. This is done by applying Delaunay triangulation.

- For each one of the Ntetra resulted tetrahedrons:

  - Calculate its volume Vi

  - Calculate the mean of our target function f at each vertex vm:  Mi=m=14fvm4 (similar to trapezoidal integration for 1D)

- Sum over the above to obtain the integral:

fx,y,zdxdydz≈i=1NtetraViMi

This method yields pretty good results compared to the results of the continuous function integration over the cylinder:

| Function | My integration | Continuous integration | Error |
| --- | --- | --- | --- |
| 1 (cylinder volume) | 7.6241 | 7.6454 | 0.28% |
| z | 1.3733 | 1.3762 | 0.21% |
| x2 | 12.8908 | 12.9207 | 0.23% |
| x2+y2∙z | 4.6411 | 4.6514 | 0.22% |

# Transition rates

## Calculation

The next step after normalizing the modes is to calculate the amplification of the spontaneous emission (Purcell factor) for mode m:

γm=Γmx,y,zΓ0

Where -  Γ0 is the spontaneous emission rate in free space. It is given in Novotny’s book as:

Γ0=ω3d23πε0εrℏc3

And we know that when assuming that the dipoles are d=|d|z , so according to Fermi’s golden rule we get the transition rate:

wme→f=2ℏε0εrd2kωmkumk,zr2δω-ωm

Assuming a single mode (k=1 only) the transition rate for a mode m would be:

Γmr,ω=wme→fρωmdωm=2ℏε0εrd2ωum,zr2δω-ωmρm(ωm) dωm=2ℏε0εrd2ωum,zr2ρm(ω)

So, for a mode m with a corresponding frequency ωm we get:

γmr,ω=2ℏε0εrd2ωum,zr2ρmωω3d23πεrε0ℏc3=6πc3ω2um,zr2ρmω

The density of states ρmω is the line-shape of spontaneous emission which is given by a normalized Lorentzian function:

ρmω=1π12Δωω-ωm2+12Δω2=1π12ωmQω-ωm2+12ωmQ2=2Qπ∙ωm4Q2ω-ωm2+ωm2

Where Q=ωmΔω. So, we get:

γmr,ω=6πc3ω2um,zr22Qπ∙ωm4Q2ω-ωm2+ωm2=12c3Qω2∙ωm(4Q2ω-ωm2+ωm2)∙um,zr2

And:

Γmr,ω=d2um,zr2ℏε0εrπ⋅4Qωmω4Q2ω-ωm2+ωm2

The layers of the devices are constructed in the following order:

d

d

z [nm]

360

8.28

1.97

8.7

3

5.35

3.38

So, I assume the dipole distribution should be:

dz=i=112uz-i⋅Δz-ζ-uz-i⋅Δz-3⋅10-9-ζ

Where -  ζ is the offset in the z axis of the first dipole layer, and Δz=30.68nm is the width of one period. And u(z) is the step function:  uz=0,  &z<01,  &z≥0

For example (when the y-axis has arbitrary values):

Notice that 3.38+5.35+3+8.7+1.97+8.28= 30.68, and 30.68⋅12=368.16 but the length of the device is 360nm. Because of the constraint which says that the layer next to the graphene should have a width of 8.28nm, I assume that the layers around z=0 in the 1st period are omitted.

We want to calculate the effective emission Γmω of the device for a specific mode m. Using the normalized dipole density Yr=Yz=dzdzdz we need to calculate:

Γmeffω=YzΓmz,ωdz=YzΓmx,y,z,ωdxdydz

This calculation is done numerically in the following steps:

- Calculating Γm for each point in space and frequency

- Splitting the space into planes with different z values

- Integrating over XY each z value plane using Delaunay triangulation and multiplying the area of each triangle by the average of Γm values of it vertices.

- Interpolating the results over the z axis.

- Multiplying by Yz and calculating the one dimensional integral

## Plotting - Regardless of the Dipole Density

The spatial distribution of γm(r, ω) for a certain frequency is proportional to um,zr2. 
Plotting the γm(r,ωm) for the modes of the device with:

- Ef=0.25eV

- R=3.0μm

- Q=10.28

The plot is on a logarithmic scale (log10γm ).

### fm=3.46THz

For γmr,ω:

For  Γmr,ωεr:

### fm=3.90THz

For γmr,ω:

For  Γmr,ωεr:

### fm=4.00THz

For γmr,ω:

For  Γmr,ωεr:

### fm=4.33THz

For γmr,ω:

For  Γmr,ωεr:

### fm=4.46THz

For γmr,ω:

For  Γmr,ωεr: